Telogy Software

BIOS MCSDK 2.0

SRIO Benchmarking Example Code User’s Guide

Applies to MCSDK release 02.00.06.18 or greater

Publication Date: November 16, 2011

Version 1.2

Texas Instruments, Incorporated
20450 Century Boulevard
Germantown, MD 20874 USA

Document License

This work is licensed under the Creative Commons Attribution-NoDerivs 3.0 Unported License. To view a copy of this license, visit http://creativecommons.org/licenses/by-nd/3.0/ or send a letter to Creative Commons, 171 Second Street, Suite 300, San Francisco, California, 94105, USA.

Contributors to this document

Copyright (C) 2011 Texas Instruments Incorporated - http://www.ti.com

Contents

1	Overview	1

2	Revision History	1

3	References	1

4	Default test control values	1

5	Test setup diagrams	2

5.1	Single EVM internally looped back	2

5.2	Single EVM externally looped back using a breakout card	2

5.3	Two EVMs connected using breakout cards	2

5.4	Single EVM looped back externally using an external SRIO switch	3

5.5	Two EVMs connected using an external SRIO switch	3

6	How latency measurements are obtained	3

6.1	Type11 latency method: (Round-trip cycles)	4

6.2	NWRITE latency method: (Round-trip cycles)	4

6.3	NREAD latency method: (End to end cycles)	4

7	Latency statistics display explanation	4

7.1	Latency header breakdown	4

8	Throughput statistics display explanation	5

8.1	Throughput header breakdown	5

9	Modifying and compiling the code for the different SRIO connection scenarios	6

9.1	Setting up C-I-C connection mode (core to core, internal loopback)	7

9.2	Setting up C-E-C connection mode (core to core, external loopback)	7

9.3	Setting up C-S-C connection mode (core to core, with a SRIO switch)	7

9.4	Setting up B-E-B connection mode (board to board, external interface)	8

9.5	Setting up B-S-B connection mode (board to board, with a SRIO switch)	8

10	Running the example code	9

10.1	Running the example code for core to core scenarios	9

10.2	Running the example code for board to board scenarios	10

11	Automating runs using the loadti that comes with Code Composer	10

11.1	Copy two enhanced java script files to your CCS loadti directory	11

11.2	Command line usage for loadti in order to run the example code	11

11.3	Workaround for argv/argc in SYS/BIOS	12

12	Sample console output	12

13	Extrapolating other statistics from the displayed output as explained with spreadsheet formulae	15

13.1	Getting the processor loading value from the statistics	15

13.2	Getting the max data rate value from the statistics	15

13.3	Getting the data throughput rate with overhead from the statistics	15

13.4	Getting the bandwidth utilization value from the statistics	15

# Overview

The SRIO benchmarking example code is created to allow customers to run benchmarks on their own TI EVMs with code that utilizes the SRIO LLD APIs. The benchmarking example code allows the user to run core to core in loopback mode (internal or external) on a single EVM, or board to board using the external interface between two EVMs. This document’s purpose is to explain how measurements are obtained and how to configure the example code for different test scenarios. SRIO physical connectivity or external SRIO switch configuration is beyond the scope of this document.

# Revision History

| Revision | Details |
| --- | --- |
| 1.0 | Initial Version |
| 1.1 | Added test setup diagrams and corrected some wording mistakes. |
| 1.2 | Updated SRIO user’s guide link and directory references for example code. |

# References

[1] Serial RapidIO (SRIO) for Keystone Devices User's Guide (Literature Number: SPRUGW1A, http://www.ti.com/lit/ug/sprugw1a/sprugw1a.pdf )

# Default test control values

The default #defines that control the example code application are located in the benchmarking.h file that is located in the “..\packages\ti\drv\srio\test\tput_benchmarking” directory of the PDK install. The default test control values are located in the section with the heading “Test control default vaules. (User changeable values)”. The benchmarking.h file contains descriptions of the various control values.

# Test setup diagrams

These are high level test setup diagrams to illustrate the different test scenarios supported by the example code.

## Single EVM internally looped back

## Single EVM externally looped back using a breakout card

## Two EVMs connected using breakout cards

## Single EVM looped back externally using an external SRIO switch

## Two EVMs connected using an external SRIO switch

# How latency measurements are obtained

For type11 and NWRITE, round-trip cycles are taken and divided by 2. For NREAD the native end-to-end cycles are measured. All measurements are taken from TX side to have the same reference clock used for timestamps. For type11 measurements, the transmit packets are prefabricated and ready to go to eliminate turnaround time. For DIO NWRITE and NREAD, all packet memory bytes are preset before the operation is started. Please note that this example code measures latency from the SRIO LLD API perspective. The example code measures LLD and hardware latency together.

## Type11 latency method: (Round-trip cycles)

- Take start timestamp

- Call sRIO LLD’s Srio_sockSend() API  (Push to TX queue)

- Call sRIO LLD's Srio_rxCompletionIsr()  (Pop RX descriptor if a packet has been  received & process the received buffer descriptor)

- Call sRIO LLD's Srio_sockRecv() API  (Copy data from the RX buffer)

- Tale end timestamp

- Compute: End to End cycles = (End timestamp - Start timestamp) / 2

## NWRITE latency method: (Round-trip cycles)

- Take start timestamp

- Call sRIO LLD's Srio_sockSend() API  (Write to LSU registers for memory write over SRIO link)

- Poll memory address of end byte to detect when entire return packet is received.

- Take end timestamp

- Compute: End to End cycles = (End timestamp - Start timestamp) / 2

## NREAD latency method: (End to end cycles)

- Take start timestamp

- Call sRIO LLD's Srio_sockSend() API  (Write to LSU registers for memory read over SRIO link)

- Poll memory address of end byte to detect when entire packet is received.

- Take end timestamp

- Compute: End to End cycles = End timestamp - Start timestamp

# Latency statistics display explanation

The following is a breakdown of the output provided for latency measurements.

## Latency header breakdown

- Core  (Core that the .out files is being run on)

- Lanes  (The number of lanes configured for the port tested also known as port width)

- Speed  (The baud rate in gigabits for which each lane is set)

- Conn  (The type of SRIO connection used)

  - C-I-C is core to core on the same EVM using internal loopback

  - C-E-C is core to core on the same EVM using external loopback

  - C-S-C is core to core on the same EVM using a external SRIO switch

  - B-E-B is board  to board using two separate EVMs using external loopback

  - B-S-B is board  to board using two separate EVMs using an External SRIO switch

- MsgType  (The type of SRIO operation used)

  - Type-11 is for a SRIO type-11 operation.

  - DIO_NW is for a SRIO directIO NWRITE operation.

  - Type-2_NR is for a SRIO directIO NREAD operation

- PktSize  (The size in bytes of the packet being transfered)

- NumPkts  (The total number of packets sent/received during the measurement)

- MnLCycs  (Minimum end to end cycles measured during latency measurement)

- AgLCycs  (Average end to end cycles measured during latency measurement)

- MxLCycs (Maximum  end to end cycles measured during latency measurement)

# Throughput statistics display explanation

The following is a breakdown of the output provided for throughput measurements.

## Throughput header breakdown

- Core  (Core that the .out files is being run on)

- Lanes  (The number of lanes configured for the port tested also known as port width)

- Speed  (The baud rate in gigabits for which each lane is set)

- Conn  (The type of SRIO connection used)

  - C-I-C is core to core on the same EVM using internal loopback

  - C-E-C is core to core on the same EVM using external loopback

  - C-S-C is core to core on the same EVM using an external SRIO switch

  - B-E-B is board  to board using two separate EVMs using external loopback

  - B-S-B is board  to board using two separate EVMs using an external SRIO switch

- MsgType  (The type of SRIO operation used)

  - Type-11 is for a SRIO type-11 operation.

  - DIO_NW is for a SRIO directIO NWRITE operation.

  - Type-2_NR is for a SRIO directIO NREAD operation

- OHBytes  (The overhead in bytes for the operation)

- PktSize  (The size in bytes of the packet being transfered)

- Pacing  (The delay cycles added to keep the RX side from getting overruns)

- Thruput  (The raw no overhead added throughput in megabits for the packet size)

- PktsSec.  (The average number of packets per second measured)

- NumPkts  (The total number of packets sent or received during the measurement)

- PktLoss  (Always “No” since the test is designed to measure without packet loss.)

- AgPCycs  (The average total number of cycles each packet took to transmit or receive)

- AgLCycs  (Average cycles the LLD took to complete)

- AgICycs  (Average number of idle cycles per packet transaction)

- AgOCycs  (Average number of cycles that were not LLD or idle cycles)

- Seconds  (The duration in seconds for measuring the packets throughput)

# Modifying and compiling the code for the different SRIO connection scenarios

The benchmarking example code allows the user to run core to core on the same EVM or to run board to board using two separate EVMs. There are settings which allow the output to indicate which mode the test is being run in as well. The default settings are contained in the benchmarking.h file. The current defaults are for core to core on the same EVM using internal loopback, the lanes are set for 5.0Gbaud and the port is configured for 4X (all four lanes are used for the port). Each connection mode shown below can be set for different lane rates and port widths.

## Setting up C-I-C connection mode (core to core, internal loopback)

- This mode is for core to core transfers on the same EVM using internal loopback. In this mode only one version of the .out file is needed. The .out file will be run on core 0 (the consumer) and core 1 (the producer).

- No modifications to the original version of the benchmarking.h file are needed to use this mode.

- The .out file should be loaded on core 0 and core 1 and run simultaneously to start benchmark measurements.

## Setting up C-E-C connection mode (core to core, external loopback)

- This mode is for core to core transfers on the same EVM using external loopback. In this mode only one version of the .out file is needed. The .out file will be run on core 0 (the consumer) and core 1 (the producer). It is assumed that the board is properly looped back over the external interface via a SMA break-out board.

- Modify the benchmarking.h file in the following way:

  - Change the  USE_LOOPBACK_MODE define from “TRUE” to “FALSE”

- Recompile the project.

- The .out file should be loaded on core 0 and core 1 and run simultaneously to start benchmark measurements.

## Setting up C-S-C connection mode (core to core, with a SRIO switch)

- This mode is for core to core transfers on the same EVM looping back through an external SRIO switch. In this mode only one version of the .out file is needed. The .out file will be run on core 0 (the consumer) and core 1 (the producer). It is assumed that the board is properly connected to the external SRIO switch and that the external SRIO switch is properly configured with the SRIO IDs, port width and lane rate. The consumer side uses 0xBE for the SRIO ID and the producer side uses 0xDE for the SRIO ID.

- Modify the benchmarking.h file in the following way:

  - Change the “IS_OVER_EXTERNAL_SRIO_SWITCH” define from “FALSE to “TRUE”, this will automatically set loopback mode to off and have the output show that the test was run using an external SRIO switch.

- Recompile the project.

- The .out file should be loaded on core 0 and core 1 and run simultaneously to start benchmark measurements.

## Setting up B-E-B connection mode (board to board, external interface)

- This mode is for board to board transfers using two separate EVMs. In this mode two different versions of the .out file are needed. The consumer .out file will be run on core 0 of the first EVM and the producer .out file will be run on core 1 of the second EVM. It is assumed that the boards are properly connected over their external interfaces via SMA break-out boards.

- Modify the benchmarking.h file in the following way for the consumer .out file:

  - Change the “IS_BOARD_TO_BOARD” define from “FALSE to “TRUE”, this will automatically set loopback mode to off and have the output show that the test was run board to board.

  - Recompile the project.

  - Save the compiled consumer .out file to a separate directory.

- Modify the benchmarking.h file in the following way for the producer .out file:

  - Change the “IS_BOARD_TO_BOARD” define from “FALSE to “TRUE”, this will automatically set loopback mode to off and have the output show that the test was run board to board.

  - Change the “CORE_TO_INITIALIZE_SRIO” define from “CONSUMER_CORE” to “PRODUCER_CORE”. This will allow the srio initialization routine to be executed for the producer EVM.

  - Recompile the project.

- Load and run the consumer .out file from the saved directory on core 0 of the first EVM. This will be the receive side for the SRIO transfer. The consumer .out file must be loaded and run before the producer .out file is run.

- Load and run the producer .out file on core 1 on the second EVM. This will be the transmit side for the SRIO transfer. This should start the transfer.

## Setting up B-S-B connection mode (board to board, with a SRIO switch)

- This mode is for board to board transfers using two separate EVMs over an external SRIO Switch. In this mode two different versions of the .out file are needed. The consumer .out file will be run on core 0 of the first EVM and the producer .out file will be run on core 1 of the second EVM. It is assumed that the boards are properly connected to the external SRIO switch and that the external SRIO switch is properly configured with the SRIO IDs, port width and lane rate. The consumer side uses 0xBE for the SRIO ID and the producer side uses 0xDE for the SRIO ID.

- Modify the benchmarking.h file in the following way for the consumer .out file:

  - Change the “IS_BOARD_TO_BOARD” define from “FALSE to “TRUE”, this will automatically set loopback mode to off and have the output show that the test was run board to board.

  - Change the “IS_OVER_EXTERNAL_SRIO_SWITCH” define from “FALSE to “TRUE”, for this scenario this define is used purely to have the output show that the test was run using an external SRIO switch.

  - Recompile the project.

  - Save the compiled consumer .out file to a separate directory.

- Modify the benchmarking.h file in the following way for the producer .out file:

  - Change the “IS_BOARD_TO_BOARD” define from “FALSE to “TRUE”, this will automatically set loopback mode to off and have the output show that the test was run board to board.

  - Change the “IS_OVER_ EXTERNAL_SRIO_SWITCH” define from “FALSE to “TRUE”, for this scenario this define is used purely used to have the output show that the test was run using an external SRIO switch.

  - Change the “CORE_TO_INITIALIZE_SRIO” define from “CONSUMER_CORE” to “PRODUCER_CORE”. This will allow the srio initialization routine to be executed for the producer EVM.

  - Recompile the project

- Load and run the consumer .out file from the saved directory on core 0 of the first EVM. This will be the receive side for the SRIO transfer. The consumer .out file must be loaded and run before the producer .out file is run.

- On the second EVM connect cores 0 and core 1. Load and run the producer .out file only on core 1 on the second EVM. This will be the transmit side for the SRIO transfer. This should start the transfer. Note: Core 0 is connected in this scenario to have the global default setup function of the GEL file run, since it only runs for core 0.

# Running the example code

This section explains how to load and run the example code.

## Running the example code for core to core scenarios

This test mode requires a single EVM.

- Power cycle the EVM to start fresh before running the throughput and latency measurements.

- Bring up CCS and connect to the platform.

- On the single EVM load the compiled .out file on both cores 0 and 1. Please see section 7 for creating the proper .out file.

- Select both cores 0 and 1 and run them simultaneously.

- Progress messages should start displaying. It will take approximately 60 minutes for all tests to complete.

## Running the example code for board to board scenarios

This test mode requires two EVMS. Two breakout boards or an external SRIO switch can be used for SRIO link connectivity. It is assumed that the physical connections on the breakout boards are correctly made. It is assumed if an external SRIO switch is used that it is correctly configured with SRIO IDs, port width, and lane rate.  Please see section 7 for the SRIO IDs used.

- Power cycle EVMs to start fresh before running the throughput and latency measurements.

- Bring up CCS and connect to the receive side platform (consumer).

- Bring up another CCS instance and connect to the transmit side platform (producer).

- On the receive side platform load the consumer .out file on core 0. Please see section 7 for creating the consumer .out file.

- On the producer side platform connect both core 0 and core 1, but only load the producer .out file on core 1. Note: Core 0 is connected in this scenario to have the global default setup function of the GEL file run, since it only runs for core 0. Please see section 7 for creating the producer .out file.

- On the receiver side platform run the consumer .out file on core 0. The consumer needs to be running before starting the producer.

- On the producer side platform run the producer .out file on core 1. The consumer needs to be running before starting the producer.

- Progress messages should start displaying. It will take approximately 60 minutes for all tests to complete.

# Automating runs using the loadti that comes with Code Composer

This section will show how to automate running the application using the “loadti” example for the DSS server that comes with Code Composer Studio. Loadti is located under the “ccsv5\ccs_base_x.x.x.xxxxx\scripting\examples\loadti” directory for CCS version 5 and located under the “ccsv4\scripting\examples\loadti” directory for CCS version 4.

## Copy two enhanced java script files to your CCS loadti directory

- Copy the java script files located in the “..packages\ti\drv\srio\test\tput_benchmarking\enhanced_loadti_files” directory of your PDK install to the loadti directory of your CCS install. Please see above for location of loadti under the CCS install directory.

- Please see the Readme.txt file in the “enhanced_loadti_files” directory for more information.

## Command line usage for loadti in order to run the example code

- Call “C:\Program Files\Texas Instruments\ccsv5\ccs_base_5.0.3.00028\scripting\examples\loadti\loadti"  -gtl "C:/workspace/evmc6678l.gel"  -gftr "Global_Default_Setup()"    -ar -t 3900000 -ctr 2 -c "c:\workspace\C6678_v5_28_mezzanine_no_gel.ccxml" "C:\Program Files\Texas Instruments\pdk_C6678_1_0_0_18\packages\ti\drv\MyexampleProjects\SRIO_TputBenchmarkingTestProject\Debug\SRIO_TputBenchmarkingTestProject.out" 1X 1.250GBaud 60seconds  >> throughput.txt"

  - Note: Always power cycle EVM before each run to start fresh.

  - Call is used since Loadti is a batch file. Call is necessary for the batch file to continue on to the next line in the batch file.

  - “C:\Program Files\Texas Instruments\ccsv5\ccs_base_5.0.3.00028\scripting\examples\loadti\loadti" is the command to start loadti.

  - -gtl "C:/workspace/evmc6678l.gel"  is the loadti option to load the GEL file. This option is not needed if the .ccxml file was created to automatically load the GEL file.

  - -gftr "Global_Default_Setup()"  is the loadti option to load the default setup from the GEL file.

  - –ar is the loadti option to do a system reset on the EVM.

  - -t 3900000 is the loadti option specifying a timeout, in this case after 65 minutes. This is used to give the application enough time to allow all the tests for this run to complete.

  - -ctr 2 is the loadti option specifying the number of core the .out file will be loaded on, in this case two cores.

  - -c "c:\workspace\C6678_v5_28_mezzanine_no_gel.ccxml" is the loadti option specifying the .ccxml file for the EVM being used. The .ccxml file should be created so that the GEL file (e.g. evmc6678l.gel or evmc6670l.gel) is automatically loaded. This can be done by clicking on the “Advanced” tab just after creating the .ccxml file.

  - "C:\Program Files\Texas Instruments\pdk_C6678_1_0_0_18\packages\ti\drv\MyexampleProjects\SRIO_TputBenchmarkingTestProject\Debug\SRIO_TputBenchmarkingTestProject.out" is the loadti option specifying the .out file that you want to load and run.

  - 1X  1.250Gbaud  60seconds  are the arguments that loadti will pass to the .out file. The options must be in the order shown. 1X is for a one lane port width (range: 1X, 2X or 4X). 1.250Gbaud is the rate at which the lanes will be set (range: 1.250Gbaud, 2.50Gbaud, 3.125Gbaud or 5.0Gbaud). 60seconds if for the amount of time in seconds to test each packet size.

  - >> throughput.txt  is a command line option used to append the console output to a file. Please note that when piping the output to a file you will not see the display. You can periodically do “type througput.txt” to see what has happened so far. You can use WordPad to view the througput.txt file. WordPad can also be used to resave the file so that it is readable with Notepad as well.

  - More information on loadti can be found at the following URL: http://processors.wiki.ti.com/index.php/Loadti

## Workaround for argv/argc in SYS/BIOS

- The error message: “SEVERE: ___c_args__ not defined.  Use "--args=n" when linking target executable to allocate n bytes for arguments.” when running the command line shown above is an indication that SYS/BIOS is not supporting argument passing. The workaround for this is to download SYS/BIOS package version 6.32.01.38 and recompile the project with this version of SYS/BIOS selected. This is only recommended as a temporary solution to get your automation started.

# Sample console output

The console output shown below is from an actual example code run. The sections highlighted in blue below are tab delimited output that can be copied from the CCS console window to a spreadsheet. The column headers were set to bold to easily locate them.

Setting core 0 test parameters to:

One 4X port

5.0Gbaud lane rate

30 seconds test time per packet size

Setting core 1 test parameters to:

One 4X port

5.0Gbaud lane rate

30 seconds test time per packet size

********************************

*********** PRODUCER ***********

********************************

WARNING: Please ensure that the CONSUMER is executing before running the PRODUCER!!

Debug(Core 1): Waiting for SRIO to be initialized.

********************************

*********** CONSUMER ***********

********************************

WARNING: Please ensure that the CONSUMER is executing before running the PRODUCER!!

Debug: Waiting for module reset...

Debug: Waiting for module local reset...

Debug: Waiting for SRIO ports to be operational...

Debug: SRIO port 0 is operational.

Debug:   Lanes status shows lanes formed as one 4x port

Debug: AppConfig Tx Queue: 0x2a0 Flow Id: 0

Debug: SRIO Driver Instance 0x@008608c0 has been created

Debug: Running test in polled mode.

Debug: SRIO Driver handle 0x8608c0.

Debug: AppConfig Tx Queue: 0x2a1 Flow Id: 1

Debug: SRIO Driver Instance 0x@008607d0 has been created

Debug: Running test in polled mode.

Debug: SRIO Driver handle 0x8607d0.

Latency: (DIO_NW, 5.000GBaud, 4X, tab delimited)

Core	Lanes	Speed	Conn	MsgType	PktSize	NumPkts	MnLCycs	AgLCycs	MxLCycs

1	4	5.000	C-I-C	DIO_NW	4	100	695	695	736

1	4	5.000	C-I-C	DIO_NW	8	100	695	695	706

1	4	5.000	C-I-C	DIO_NW	16	100	695	697	711

1	4	5.000	C-I-C	DIO_NW	32	100	737	740	755

1	4	5.000	C-I-C	DIO_NW	64	100	785	799	811

1	4	5.000	C-I-C	DIO_NW	128	100	920	922	935

1	4	5.000	C-I-C	DIO_NW	256	100	1158	1161	1175

1	4	5.000	C-I-C	DIO_NW	512	100	1325	1325	1339

1	4	5.000	C-I-C	DIO_NW	1024	100	1623	1625	1640

1	4	5.000	C-I-C	DIO_NW	2048	100	2225	2235	2240

1	4	5.000	C-I-C	DIO_NW	4096	100	3454	3455	3456

1	4	5.000	C-I-C	DIO_NW	8192	100	5885	5885	5899

Throughput: (RX side, DIO_NW, 5.000GBaud, 4X, tab delimited)

Core	Lanes	Speed	Conn	MsgType	OHBytes	PktSize	Pacing	Thruput	PktsSec.	NumPkts 	PktLoss	AgPCycs	AgLCycs	AgICycs	AgOCycs	Seconds

0	4	5.000	C-I-C	DIO_NW	16	4	0	78.43	2450980.50	73600000	No	408	42	337	29	30.03

0	4	5.000	C-I-C	DIO_NW	16	8	0	156.86	2450980.50	73600000	No	408	42	336	30	30.03

0	4	5.000	C-I-C	DIO_NW	16	16	0	313.73	2450980.50	73600000	No	408	42	336	30	30.03

0	4	5.000	C-I-C	DIO_NW	16	32	0	627.45	2450980.50	73600000	No	408	42	336	30	30.03

0	4	5.000	C-I-C	DIO_NW	16	64	0	1254.90	2450980.50	73600000	No	408	42	336	30	30.03

0	4	5.000	C-I-C	DIO_NW	16	128	0	2509.80	2450980.50	73600000	No	408	42	336	30	30.03

0	4	5.000	C-I-C	DIO_NW	16	256	0	5019.61	2450980.50	73600000	No	408	42	337	29	30.03

0	4	5.000	C-I-C	DIO_NW	16	512	0	10039.22	2450980.50	73600000	No	408	42	337	29	30.03

0	4	5.000	C-I-C	DIO_NW	16	1024	0	12941.55	1579778.88	47400000	No	633	42	561	30	30.00

0	4	5.000	C-I-C	DIO_NW	16	2048	0	13159.84	803212.88	24200000	No	1245	42	1173	30	30.13

0	4	5.000	C-I-C	DIO_NW	16	4096	0	13271.77	405022.28	12200000	No	2469	42	2397	30	30.12

0	4	5.000	C-I-C	DIO_NW	16	8192	0	13282.53	202675.31	6200000	No	4934	42	4863	29	30.60

Throughput: (TX side, DIO_NW, 5.000GBaud, 4X, tab delimited)

Core	Lanes	Speed	Conn	MsgType	OHBytes	PktSize	Pacing	Thruput	PktsSec.	NumPkts 	PktLoss	AgPCycs	AgLCycs	AgICycs	AgOCycs	Seconds

1	4	5.000	C-I-C	DIO_NW	16	4	0	78.43	2450980.50	73600000	No	408	326	25	57	30.03

1	4	5.000	C-I-C	DIO_NW	16	8	0	156.86	2450980.50	73600000	No	408	326	25	57	30.03

1	4	5.000	C-I-C	DIO_NW	16	16	0	313.73	2450980.50	73600000	No	408	326	25	57	30.03

1	4	5.000	C-I-C	DIO_NW	16	32	0	627.45	2450980.50	73600000	No	408	326	25	57	30.03

1	4	5.000	C-I-C	DIO_NW	16	64	0	1254.90	2450980.50	73600000	No	408	326	25	57	30.03

1	4	5.000	C-I-C	DIO_NW	16	128	0	2509.80	2450980.50	73600000	No	408	326	25	57	30.03

1	4	5.000	C-I-C	DIO_NW	16	256	0	5019.61	2450980.50	73600000	No	408	326	25	57	30.03

1	4	5.000	C-I-C	DIO_NW	16	512	0	10039.22	2450980.50	73600000	No	408	326	25	57	30.03

1	4	5.000	C-I-C	DIO_NW	16	1024	171	12941.55	1579778.88	47400000	No	633	326	248	59	30.00

1	4	5.000	C-I-C	DIO_NW	16	2048	784	13159.84	803212.88	24200000	No	1245	326	861	58	30.13

1	4	5.000	C-I-C	DIO_NW	16	4096	2009	13271.77	405022.28	12200000	No	2469	326	2085	58	30.12

1	4	5.000	C-I-C	DIO_NW	16	8192	4475	13279.84	202634.25	6200000	No	4935	326	4551	58	30.60

Latency: (DIO_NR, 5.000GBaud, 4X, tab delimited)

Core	Lanes	Speed	Conn	MsgType	PktSize	NumPkts	MnLCycs	AgLCycs	MxLCycs

1	4	5.000	C-I-C	DIO_NR	4	100	924	953	995

1	4	5.000	C-I-C	DIO_NR	8	100	924	953	995

1	4	5.000	C-I-C	DIO_NR	16	100	924	956	995

1	4	5.000	C-I-C	DIO_NR	32	100	924	990	1017

1	4	5.000	C-I-C	DIO_NR	64	100	924	1049	1070

1	4	5.000	C-I-C	DIO_NR	128	100	924	1200	1227

1	4	5.000	C-I-C	DIO_NR	256	100	924	1420	1428

1	4	5.000	C-I-C	DIO_NR	512	100	924	1577	1632

1	4	5.000	C-I-C	DIO_NR	1024	100	924	1902	1920

1	4	5.000	C-I-C	DIO_NR	2048	100	924	2477	2545

1	4	5.000	C-I-C	DIO_NR	4096	100	924	3750	3774

1	4	5.000	C-I-C	DIO_NR	8192	100	924	6595	6597

Throughput: (TX side, DIO_NR, 5.000GBaud, 4X, tab delimited)

Core	Lanes	Speed	Conn	MsgType	OHBytes	PktSize	Pacing	Thruput	PktsSec.	NumPkts 	PktLoss	AgPCycs	AgLCycs	AgICycs	AgOCycs	Seconds

1	4	5.000	C-I-C	DIO_NR	28	4	0	46.78	1461988.25	44000000	No	684	339	310	35	30.13

1	4	5.000	C-I-C	DIO_NR	28	8	0	93.57	1461988.25	44000000	No	684	339	310	35	30.13

1	4	5.000	C-I-C	DIO_NR	28	16	0	184.97	1445086.75	43400000	No	692	331	325	36	30.05

1	4	5.000	C-I-C	DIO_NR	28	32	0	351.65	1373626.38	41200000	No	728	336	356	36	30.02

1	4	5.000	C-I-C	DIO_NR	28	64	0	647.28	1264222.50	38000000	No	791	338	416	37	30.07

1	4	5.000	C-I-C	DIO_NR	28	128	0	1122.81	1096491.25	33000000	No	912	338	538	36	30.12

1	4	5.000	C-I-C	DIO_NR	28	256	0	1756.43	857632.94	25800000	No	1166	342	788	36	30.09

1	4	5.000	C-I-C	DIO_NR	28	512	0	3119.57	761614.63	23000000	No	1313	339	939	35	30.22

1	4	5.000	C-I-C	DIO_NR	28	1024	0	5063.04	618047.00	18600000	No	1618	340	1242	36	30.10

1	4	5.000	C-I-C	DIO_NR	28	2048	0	7360.29	449236.31	13600000	No	2226	338	1852	36	30.28

1	4	5.000	C-I-C	DIO_NR	28	4096	0	9359.61	285632.69	8600000	No	3501	337	3128	36	30.11

1	4	5.000	C-I-C	DIO_NR	28	8192	0	10379.47	158378.20	4800000	No	6314	341	5937	36	30.31

Latency: (Type-11, 5.000GBaud, 4X, tab delimited)

Core	Lanes	Speed	Conn	MsgType	PktSize	NumPkts	MnLCycs	AgLCycs	MxLCycs

1	4	5.000	C-I-C	Type-11	16	100	1482	1552	1620

1	4	5.000	C-I-C	Type-11	32	100	1542	1564	1585

1	4	5.000	C-I-C	Type-11	64	100	1621	1648	1672

1	4	5.000	C-I-C	Type-11	128	100	1806	1819	1872

1	4	5.000	C-I-C	Type-11	256	100	2109	2153	2161

1	4	5.000	C-I-C	Type-11	512	100	2170	2232	2277

1	4	5.000	C-I-C	Type-11	1024	100	2422	2498	2589

1	4	5.000	C-I-C	Type-11	2048	100	3114	3188	3223

1	4	5.000	C-I-C	Type-11	4096	100	4534	4587	4675

Throughput: (RX side, Type-11, 5.000GBaud, 4X, tab delimited)

Core	Lanes	Speed	Conn	MsgType	OHBytes	PktSize	Pacing	Thruput	PktsSec.	NumPkts 	PktLoss	AgPCycs	AgLCycs	AgICycs	AgOCycs	Seconds

0	4	5.000	C-I-C	Type-11	24	16	0	164.10	1282051.25	38600000	No	780	628	0	152	30.11

0	4	5.000	C-I-C	Type-11	24	32	0	326.53	1275510.25	38600000	No	784	633	0	151	30.28

0	4	5.000	C-I-C	Type-11	24	64	0	652.23	1273885.38	38400000	No	785	634	0	151	30.15

0	4	5.000	C-I-C	Type-11	24	128	0	1304.46	1273885.38	38400000	No	785	634	0	151	30.15

0	4	5.000	C-I-C	Type-11	24	256	0	2635.78	1287001.25	38600000	No	777	626	0	151	29.99

0	4	5.000	C-I-C	Type-11	24	512	0	5211.20	1272264.63	38200000	No	786	635	0	151	30.05

0	4	5.000	C-I-C	Type-11	24	1024	0	10422.39	1272264.63	38000000	No	786	635	0	151	29.90

0	4	5.000	C-I-C	Type-11	24	2048	0	11636.36	710227.25	21400000	No	1408	632	624	152	30.13

0	4	5.000	C-I-C	Type-11	24	4096	0	11640.50	355239.78	10800000	No	2815	631	2033	151	30.41

Throughput: (TX side, Type-11, 5.000GBaud, 4X, tab delimited)

Core	Lanes	Speed	Conn	MsgType	OHBytes	PktSize	Pacing	Thruput	PktsSec.	NumPkts 	PktLoss	AgPCycs	AgLCycs	AgICycs	AgOCycs	Seconds

1	4	5.000	C-I-C	Type-11	24	16	244	164.10	1282051.25	38600000	No	780	108	607	65	30.11

1	4	5.000	C-I-C	Type-11	24	32	244	326.53	1275510.25	38600000	No	784	108	612	64	30.28

1	4	5.000	C-I-C	Type-11	24	64	246	652.23	1273885.38	38400000	No	785	108	612	65	30.15

1	4	5.000	C-I-C	Type-11	24	128	244	1304.46	1273885.38	38400000	No	785	108	613	64	30.15

1	4	5.000	C-I-C	Type-11	24	256	234	2635.78	1287001.25	38600000	No	777	108	604	65	29.99

1	4	5.000	C-I-C	Type-11	24	512	246	5211.20	1272264.63	38200000	No	786	108	614	64	30.05

1	4	5.000	C-I-C	Type-11	24	1024	244	10422.39	1272264.63	38000000	No	786	108	613	65	29.90

1	4	5.000	C-I-C	Type-11	24	2048	0	11644.63	710732.06	21400000	No	1407	108	1242	57	30.13

1	4	5.000	C-I-C	Type-11	24	4096	0	11640.50	355239.78	10800000	No	2815	108	2650	57	30.41

# Extrapolating other statistics from the displayed output as explained with spreadsheet formulae

This section show spreadsheet formulas to get processor loading, max data rate for the links lanes and lane speed combination, throughput data rate with overhead included, and bandwidth utilization from the statistics output by this example code. The formulas would be used for each packet size measured. Items shown below in bold denote that this is a column of the statistics shown in bold blue above. The value within the brackets [] can be copied to the spreadsheet.

## Getting the processor loading value from the statistics

- Spreadsheet formula for getting the processor loading percentage: [ = 1 - $O5/$M5 ]

- Cell $O5 is the average idle cycles (AgICycs). Cell $M5 is the average packet cycles (AgPCycs).

## Getting the max data rate value from the statistics

- Spreadsheet formula for getting the max data rate in Mbps: [ =$B5 * $C5 * 0.8 * 1000 ]

- Cell $B5 is the number of lanes (Lanes). Cell $C5 is the lane speed in Mbps (Speed). The .8 value in the equation accounts for the 20% overhead taken by the SRIO 8b/10b hardware encoding/decoding method. The 1000 value is to represent result in Mbps.

## Getting the data throughput rate with overhead from the statistics

- Spreadsheet formula for getting throughput rate in Mbps with packet overhead included:                   [ =(((FLOOR((($G5-1)/256)+1,1) * $F5 * 8) + ($G5 * 8)) * $J5) / 1000000 ]

- Cell $G5 is the packet size (PktSize). Cell $F5 is packet overhead in bytes (OHBytes). Cell $J5 is the packets per second (PktsSec.). This formula accounts for the fact that the maximum MTU size for SRIO is 256Bytes. This means that the overhead must be applied to every 256Byte segment transmitted. So for a packet with a size of 1024Bytes, the packet will need to be transmitted in four 256Byte segments and each segment will need to have the overhead applied to it.

## Getting the bandwidth utilization value from the statistics

- Spreadsheet formula for getting bandwidth utilization percentage: [ = $T5/$S5 ]

- Cell $T5 is the value derived from the calculation (in section 13.3) to get the throughput with overhead. Cell $S5 is the value derived from the calculation (in section 13.2) to get the max data rate.